SZD-55 „Promyk“

Building instructions for part - finish at home,

or step by step

Kit finish at home contains this parts to finalization model airplane:

For model finish at home - Nr.: 11101 to landing gear:

Fibreglass case air-brakes, two-sided glue bands, air-brakes, piping into fuselage, steering link servo, click hinge, laminated piece.

Landing gear, fibreglass bands, elastic band, wiry eye, bag with micro-balons.

For model finish at home - Nr.: 10101 void landing gear:

Fibreglass case air-brakes, two-sided glue bands, air-brakes, piping into fuselage, steering link servo, click hinge, laminated piece, bag with micro-balons.

For finishing the kit you need ( not included in the set) :

sharp knife, scissors, cyanoacrylate glue, epoxyd glue, drilling machine, needle files, screwdriver, emery-paper, bicarbonate,  scroll saw, handsaw, set-square, kleště, pair of tongs, abrasive disk,  brush-pencil.

FINISHING THE KIT

The kit is not suitable for the beginners, the workflow can be modified as required according to one’s experience.

COCKPIT

1. Appropriate to cockpit.

2. Sharpen, deburr toll cockpit.

4. Sharpen fray cockpit.

3. Sharpen fray cockpit.

RUDDER FLAP

5. Appropriate to rudder flap.

6. Prune of burr.

8. Fit and sink trailing edge rudder flap.

7. Prune of rudder flap.

9. And we've rady rudder flap.

FUSELAGE

11. On-click hinge on rudder flap.

10. Prepare rudder flap, laminated piece and click hinge.

12. Laminated piece set to click hinge.

13. Define spacer among laminated piece and rudder flap on cca 3 mm.

14. Seat to the plains.

15. Concoction epoxyd with mixture micro-balons, which are in bag delivery with kit.

17. On two click hinge.

16. Dab mixture on click hinge.

18.  Leave 24 clock around setting.

20. Dab mixture on laminated piece.

19. Concoction epoxyd with mixture micro-balons, which are in bag delivery with kit.

21. Laminated part introduction in rudder and guarantee against shift.

23. Execute verification flap angle to the left.

22. Execute verification flap angle to the right.

24.  Leave 24 clock around setting.

25. Mark point for opening wire servo.

26. Drill hole for wire servo.

27. Sharpen hole for wire servo.

29. Drill hole.

28. Mark points for hole.

30. Drawing linked hole.

31. Drawing link all hole.

32. Cut out, milling opening for servo.

33. Opening fit and sink emery-paper.

35. Clip off band fibreglass.

34. And we've ready opening for servo.

37. Clip off on requisite length.

38. This way yourself setting-up two bands.

39. Dab second cyanoacrylate glue.

36. Size up st. tape length for landing gear well.

41. Press and leave get dry.

40. Insertion band into landing gear well.

42. It like get up on the other side.

43. And we've glued-on.

44. Prepare wiry eye, belong kit.

45. Dab drop cyanoacrylate glue.

47. Fill up bicarbonate.

46. Wiry eye give into drops cyanoacrylate glue.

48. Prepare landing gear.

49. Landing gear set in landing gear well and centered.

50. Landing gear secure drop cyanoacrylate glue.

51. Prepare wing joiner, piping into fuselage, cockpit and bag with micro-ballon.

53. Polish wing joiner, which is already dissociation.

52. Polish wing joiner, which is already dissociation.

54. Ends piping sharpen.

55. Deburr piping.

56. Ends piping sharpen.

57. Deburr piping.

59. Subsequently set prominent piping on wing joiner.

58. Into fuselage set hind piping on wing joiner.

60. Dismantling bar landing gear.

61. Protrusion bogie-wheel and we've ready landing gear to laminating.

62. Fibreglass bands snip on length c. 20 cm.

63. Concoction epoxyd with mixture micro-balons, which are in bag delivery with kit.

65. Dab mixture piping and fuselage.

64. Concoction mixture epoxyd and micro-ballon.

66. Dab mixture on all piping linked to fuselage.

67. Mixture stuff spacer intermediate fuselage and landing gear.

68. Place glass fibre bands and clean epoxyd permeate through.

69. Permeate through glass fibre bands.

71. Cockpit set to fuselage and check, whether we're fuselage  no deform.

70. It like work also on the other side.

72. Leave 24 clock around setting.

73. End wing joiner grip in pincers and twitch turn about axes.

75. Prepare elastic band and tweezer.

74. End wing joiner grip in pincers and twitch turn about axes.

77. Run elastic band infra-axillary landing gear.

76. Spread, open tweezers wiry eye.

78. Run elastic band and take up.

79. Elastic band set in wiry eye.

80. Ensure, press wiry eye.

The wing is finished now.

FUSELAGE

WING

82. Draw off capping pass bands.

81. Prepare scalpel, brake plate and two-sided glue bands.

83. Sticking brake plate on bands.

84. Cut pursuant to st. brake plate.

85. This identical make with second brake plate.

86. A we've ready both brake plate.

88. Drill hole for wire servo.

87. Mark point for opening wire servo.

89. Sharpen hole for wire servo.

90. Prepare handsaw to trim away aileron.

91. Incision aileron on some ends.

92. Incision aileron on second ends.

94. And on second ends.

93. Turn wing on underside and incision aileron on some ends.

95. Canny fold down aileron and move hinge.

96. Prepare sand-paper.

98. Inspect deflection aileron on both sides.

97. Sharpen gap among wings and aileron, so with a view to aileron well move.

100. Mark points for hole.

99. sand down toll aileron.

101. Drill hole.

102. Drawing link all hole.

103. Cut out, milling opening for servo.

104. Milling of all four opening for servo.

106. You-mill opening for air-brakes.

105. Opening fit and sink emery-paper.

107. You-mill opening for air-brakes.

108. You-mill opening for air-brakes.

109. You-milled opening for air-brakes.

110. Scalpel into-cut ends.

112. Clean emery-paper toll.

111. Take out cull fibreglass.

113. Clean emery-paper toll.

114. Prepare air-brakes and steering link servo.

115. Set steering link into air-brakes.

116. Set steering link into both air-brakes.

118. In this way prepared both air-brakes.

117. Pincers pressing, ensure end steering link.

119. Small-bladed screwdriver click first off hither hinge.

120. And then hind hinge.

121. This way prepare both air-brakes.

122. Prepare scissors, sand-paper and fibreglass case air-brakes.

124. Sand-paper fit and sink fibreglass case air-brakes.

123. Scissor fibreglass case air-brakes.

125. This way prepared both fibreglass case air-brakes.

126. On wings inspect seat, case air-brakes.

127. Draw off capping pass bands.

128. For certainty, insure cyanoacrylate.

130. Press ends case to air-brakers.

129. Stick case air-brakers, that way so as to fit with opening in wings.

131. Push out air-brakers into get dry glue.

132. A we've down.

Part, finish at home, we've ready and next go ahead pursuant to st. building instructions.