SZD – 55 „Promyk“

Purchasing this kit, you got a semi-scale of  a polish sport glider.The model is designed for slope soaring and is recommended first af all for experienced model pilots ,who you will appreciate it´s flying characteristics. Equipped with the tow hook the model is good for thermal soaring, too. Finally it can also be motorized, loosing the scale look in this case.

An up-to-date technology - sandwich - is used in this construction.

To finish the model you need (not included in the kit) :

- sharp knife

- scissors

- cyanoacrylate adhesive

- drilling machine

- fine files

- screwdriver

- microbaloons or similar material

To fly you need (not included in the kit) :

4 channel RC set

RC function:                                             	 recommended servos:

#### Elevator		1 pc, micro servo

Rudder		1 pc, micro servo

Ailerons	                                                     	2 pcs, micro servo

#### Air brakes	2 pcs, micro servo

Landing gear (if installed)                         	1 pc, landing gear servo

Tow hook (not included in the kit)	            1 pc, optional servo

###### Kit includes / list of parts

- fiberglass composite fuselage with glued stabilizer trailing edge, glued landing gear (if kit version with landing gear)

- fiberglass composite wing

- fiberglass composite horizontal stabilizer

- fiberglass composite rudder

- cockpit

- canopy

- wing joiner

- transfer-picture

- accessorries

- building instructions

WARNING !

You are fully responsible for safety when using this model,improper use of this product can lead to bodily harm or property damage. Fly it far enough from built- up areas. Be sure nobody else uses the same frequency as you do.

For age of 14 and up. To be used under a supervision of an adult person.

Don´t leave the model in the areas of a high temperature, such as inside a closed car or on the direct sunlight. Temperatures above 40°C (104°F) might decrease the strength of the model components.

A brief history of the full size glider :

In 1989, a Polish pilot, T.Rubaj, won the gold medal at the Junior European Gliding Championship flying the first prototype of this glider. Two years later, J.Trzeciak won the silver medal at the XXII. World Championship flying a serial production glider.

FINISHING THE MODEL

The kit is not expected to be used by beginners, thus the building instructions can be modified as you prefer.

##### Fuselage

Install the elevator and rudder servos into the vertical stabilizer.

- Elevator – adjust the neutral position of the servo fitted with the servo lever and the    pushrod and glue it to the rear side of the vertical stabilizer using the double-sided adhesive tape.

- Rudder – according to the servo size use a piece of  balsa to adjust the servo height so, that the servo lever is out enough above the stab surface.Glue the balsa piece using cyanoacrylate.Use clear nitro varnish to paint the balsa surface, preferably 2 layers for better adhesion of the adhesive tape. Glue in the rudder servo.

- Landing gear – if to be installed, use cyanoacrylate to glue the spruce blocks into the fuselage as the landing gear servo holders. For better contact use the following way: screw the servo to the holders, insert everything to the proper place and fix using small cyanoacrylate drops.Unscrew the servo and fix the blocks using microballoons and thin cyanoacrylate to ensure a good connection.

- Receiver and batteries installation – it is suggested to use foam to protect the receiver from vibrations and impacts and to fix the components against the movement. Connect all components together.

# Cockpit and canopy

The parts are delivered separately so that everybody can finish the cockpit as required.

- Glueing the parts together – we suggest to insert the finished cockpit into the rough      canopy part and step by step glue together using cyanoacrylate (see fig.1).Cut to the right size using a sharp knife.

- Canopy lock – there are 2 possibilities :

a/ standart lock installation, loosing a bit of scale look due to the lock lever.

b/ using the microbaloons and cyanoacrylate glue a formed ( see fig.2) piece of 1mm steel wire to the rear cockpit wall.Use a rubber loop to fix the canopy/cockpit in place not loosing the scale look in this case.

see fig.1

see fig.2

Wing

- Air-brakes - insert the servo cable into the wing up to the air-brake servo cutout. Connect and adjust the servo with the horn and the pushrod installed and fix the servo using a piece of a double-sided adhesive tape.

- Ailerons - insert the servo cable into the wing up to the aileron servo cutout. Connect and adjust the servo neutral position with the horn and the pushrod installed and fix the servo using a piece of a double-sided adhesive tape.

- Servo covers – fix using a piece of a double-sided adhesive tape.

# Finishing and flying

- Decorative transfer-picture – cut the transfer overlapped. Wet the fuselage slightly using soap solution and apply the transfer. When in proper position, wipe out using a piece of clean cloth and let dry.

- Center of gravity – check the C.G. position (it should be 95 - 105 mm from the leading edge) and the proper functionality and throws of all control surfaces.

- Flying – try hand launching against the wind. This model should not be used by beginners, it requires certain amount of experience and continual awareness.

Recommendet deflections of the cotrol surfaces:

Ailerons:			+ 10 - 14 mm

–   0 – 10 mm

## Elevator:  			+ 10 mm

## – 10 mm

Rudder: 			+ max.

– max.

Have a lot of fun with your SZD-55 !

Basic technological specifications :

Fuselage – made of  fibreglass composite, coloured with white gelcoat .

Wing –  manufactured using a modern sandwich technology with carbon fibre main spars, coloured with white ( red tips ) gelcoat ; crescent-shaped leading edge

Horizontal stabilizer – manufactured using a modern sandwich technology with carbon fibre main spars, coloured with white gelcoat.

Rudder  – made of  fibreglass composite, coloured with white gelcoat.

Cockpit – made of  fibreglass composite, coloured with grey gelcoat.

Technical data:

Original	Model

scale		1:1	1:5

length		6,85 m	1370 mm

wingspan	15 m	3000 mm

height		1,47 m	294 mm

wing area	9,6 m2	38,08 dm2

taper		23,44	23

dihedral		3°	2,5°

weight		215 – 500 kg	2100 – 2350 g

wing loading	28,5 – 52,1 kg/m2	55,15 – 61,71 g/dm2

max. speed	250 km/h	90 km/h

min. speed	63 km/h	28 km/h

slide coefficient	44,1	21,5 – 22,5

airfoil		NN – 27	S 3021